ANSFORD MANUFACTURING GROUP

Process Capability Study Plan

____________________________________________________________________

Part / drawing:  PB-1140 pump body - drawing PB-1140 rev E

Characteristic:  Spigot OD 12.000 mm (LSL 11.940 / USL 12.060) - critical characteristic

Line / station:  Line 2, in-cell finishing (machining) station

Measuring equipment:  CMM asset AMG-CMM-02

Customer requirement:  Harlow Fluid Systems Ltd, ref HFS-QR-114 (form QF114)

Prepared by:  Priya Nair, Quality Engineer - 15/05/2026

Approved by:  Sarah Bright, Quality Manager - 15/05/2026

____________________________________________________________________

1. Purpose

To gather capability evidence for the PB-1140 spigot OD to support Harlow specification sign-off under requirement HFS-QR-114.

2. Method

One subgroup of 5 consecutive parts is taken each production morning (approximately 07:45) from the Line 2 finishing station.

Sampling runs over 30 production days, starting w/c 18/05/2026, under normal production conditions.

Parts are measured the same day on AMG-CMM-02 by the quality technician.

Spigot OD values are recorded to 0.001 mm on the study record sheet.

3. Reporting

On completion, results are transferred to Harlow form QF114, reporting the grand mean, the mean range, σ̂ = R̄/d₂, Cp and Cpk. The completed QF114 is submitted together with the gauge calibration certificate and the R&R summary.

4. Acceptance

Cpk ≥ 1.33 per HFS-QR-114.

____________________________________________________________________

Prepared: P. Nair          Approved: S. Bright          Date: 15/05/2026